Every bread is a story and an art expression. To create a bread, it is not enough to mix all the ingredients, but every element and movement will lead to the development of a complex system which tells a story to our sensors.

Sourdough fermentation systems

INTRODUCTION

Sourdough is a flour – water mix let to ferment at an optimal temperature for a period of time, in which the microorganisms are reproducing and release special nutritive and aroma compounds.

In order to produce the sourdough used in the bread making process, it is necessary to use exact proportions of starter (mature sourdough), flour and water.

The obtained mix is let to ferment until an optimum between yeast and lactic acid bacteria is achieved; after this period, the sourdough can be directly used to make bread, or it can be cooled to retain its optimal values in order to be used for a longer period of time.

The process of making sourdough is influenced by the quality of the materials used, the environmental variations (especially temperature) and by the cleanliness of the process. So, in order to obtain the optimal results, the producing and storing conditions need to be observed with precision.

The ability and experience of the operator are not enough to guarantee the stability and efficiency of sourdough, especially in a high scale process.

WORKING PRINCIPLE FOR SOURDOUGH FERMENTATION SYSTEM

- Mixing phase: The resulting product is homogeneously distributed with the help of specific equipment dedicated for mixing the ingredients.

- Fermentation phase: After mixing, is starts the fermentation phase, where the temperature is kept constant at 25 – 26 °C, by the refrigeration unit, and the pH value of sourdough is continuously monitored with the help of an immersed electrode. This sends information regarding fermentation activity of yeast and bacteria which release o series of organic acids, lactic acid being the predominant one.

- Obtaining the sourdough: Considering the desired properties of the final product, a predefined pH value is introduced, which can vary between 3.8 and 4.6. The necessary time to obtain a mature sourdough (ready for use) depends on the established pH value, and varies between 3.5 and 6 hours. Once the desired pH is achieved (the sourdough has matured enough), the refrigeration unit is automatically activated and the system enters the conservation – keeping period of the sourdough, which is cooled up to 4 – 8 °C and can be kept up to 3 – 4 days without alteration.

- During the fermentation and conservation periods, the sourdough mass is automatically stirred with the help of the mixing equipment, which keeps homogeneous physio – chemical characteristics through the entire mass. The cycles for mixing – fermentation – conservation are automated and do not request an operator assistance.

- The technological process is has a three phase stage, which starts with a starter, continues with a sourdough phase and the obtained product is used with other ingredients to obtain the final dough.

- The cooled sourdough is transferred to the dough kneading area with the help of a screw pump. Its transportation capacity is approximately 35 l/min, which means 40 kg /minute.

ADVANTAGES FOR USING THE SOURDOUGH FERMENTATION SYSTEM

- Constant productive quality

- Higher process efficiency with low operating

- Compliance to sanitarium rules with no contamination possibility

APPLICATIONS:

Bread making, pastry

ADVANTAGES FOR SOURDOUGH USAGE IN FINISHED PRODUCTS:

- Improving the structure of the finished product

- The growth of dough extensibility

- Reducing/ eliminating technological improvers

- Improving the freshness period of the finished product

- High resistance to mold and bacteria development

- Improving flavor and smell of the finished product

- Reducing the interaction of phitic acid

Technical specifications for sourdough fermentation system

Functions:

- Mix a quantity of flour and water with one of starter;

- The fermentation process is controlled with automatization unit;

- The fermentation process is controlled with: fermentation time, pH sourdough value, temperature;

- After fermentation period, the system can automatically dose the requested quantity of sourdough;

- The pH value of sourdough is continuously monitored.

Technical specifications for sourdough fermentation system and component parts

| Characteristic | Value |
| --- | --- |
| Storage capacity | 3000 L |
| Material in storage | Wheat dough |
| Reservoir material | Aisi Steel 316 |
| Exterior surface | 2B |
| Interior surface | 2B |
| Wall scraper | Composite |
| Water dosage | Automated |
| Mixing mechanism | Slow start, right/left (power: 4,5 kW) |
| Evacuation capacity | 30 L/min |
| Evacuation system | Pump |
| Partial product discharge | DN50 with conic nozzle |
| Total product discharge | DN50 with conic nozzle |
| Cleaning | Manual |
| Weighing and dosage precision | 300 g/600g |
| Electrical command unit | External |
| Power line | 400V/16A – 25A |
| Other elements | Ladder support |
|  | Entrance support |
|  | Probe valve |

Dimensions for sourdough storage tank

| Characteristic | Value |
| --- | --- |
| Reservoir | V=3 m3 |
|  | D =1273 mm |
|  | Hcil=2250 mm |
| Roof sight hole (DN300) | H=200 with thermal insulation |
| Admission/evacuation valve | 30m / 11/4” |
| Cooling coat | D=1273 mm |
|  | F=1.9 m2 |
|  | H=500mm |
|  | Pmax=2.5 bar, G1” |
|  | L=200mm |
| Helicoidally sight hole | 450x320mm with thermal insulation |
| Polyurethane insulation | 80mm with steel plate |

Technical specifications for screw pump

| Characteristic | Value |
| --- | --- |
| Pumped liquid | Wheat dough (flour+water) |
| Liquid viscosity | 5000 mPa |
| Liquid temperature | Ambient |
| Liquid density | 1200 kg/m3 |
| Pumped debit | 2m3/h |
| Working pressure | 2 bar |
| Aspiration pressure | Positive ( the liquid enters gravitationally into the pump) |
| Aspiration/discharge | Driving plate DN50/DN50 accordingly DIN 11851 |
| Shaft pump speed | 155 rpm |
| Engine power | 1.5 kW |
| Engine speed | 1500 rpm |
| Driving voltage | 400V – 50 Hz |
| Engine protection degree | IP55 |
| Engine insulation class | F |

Pump materials

| Characteristic | Value |
| --- | --- |
| Pump body | Stainless steel AISI 316 |
| Rotor | Stainless steel AISI 316 |
| Stator | NBR food accepted |
| Mechanical seal | SiC/SiC/EPDM + 3L steel tank (P=6bar) for mechanical seal lubrication, Stainless steek AISI 307L |

Ambient temperature

| Element | Characteristic | Value |
| --- | --- | --- |
| Cooling | Cooling capacity | 28.6 kW |
|  | Absorbed power | 10.7 kW |
| Heating | Heating capacity | 36.7 kW |
|  | Absorbed power | 11.8 kW |
| Compressor | Number | 1 |
|  | Type | Scroll |
| Evaporator | Water debit | 1.37 L/s |
|  | Pressure loss | 51 kPa |
|  | Hydraulic nipples | G1” |
| Capacitor | Fans | 2 |
|  | Air flux | 4.4 m3/s |
| Electrical characteristics | Driving voltage | 400/3+N/50, V/Ph/Hz |
|  | Maximum current in function | 28A |
|  | Maximum current at start point | 123A |
| Hydraulic circuit | Nominal pump power | 0.55 kW |
|  | Specific pump pressure | 181 kPa |
|  | Expansion tank | 5L |
|  | Hydraulic nipples | G1” |
| Sonic pressure | STD | 52 dB |
| Weight | Transportation weight | 245 kg |
|  | Functioning weight | 248 kg |

Chiller dimensions

| Characteristic | Value |
| --- | --- |
| L | 1850 mm |
| P | 1000 mm |
| H | 1300 mm |